A proposal for participation in the FP6 programme for neutrons software (continuation of SCANS) of the Grenoble group (INFM-ILL-ESRF)

1) Current activity

This document describes briefly an additional proposal for the development and continuation of software development for neutron optics and instrumentation. The goal is to continue and further extend the activities initiated in the SCANS project.

The "Grenoble" group is composed by a collaborative action between several groups located at the Polygone Scientifique in Grenoble, mainly: the Istituto Nazionale di Fisica della Materia (INFM), the Institute Laue-Langevin (ILL) and the European Synchrotron Radiation Facility (ESRF).

SCANS has supported the Grenoble group in funding:

- A 12 months Post-Doc (Christophe Boisseau, April 2001-March 2002)

- A 18 months Post-Doc (Lucia Alianelli, October 2002-March 2004)

- A 12 months Post-Doc (selection of the candidate is presently carried out)

The main activity of the group was to extend to neutrons the experience in the development of programs dedicated to the description of optical elements and complete beamlines, already established for the case of synchrotron beams.

This activity has been focused on the development of models and codes describing the optical components starting from physical principles, and on validating the methods with a comparison with experiments. The present aim is to complement existing ray-tracing programs (mainly McStas and Vitess) with other modules. On one side, we have developed stand-alone programs that describe analytically the diffraction by neutron components as crystals and multilayers. On the other hand, we have created Monte Carlo modules (imperfect crystals) based on an accurate physical description. These modules have been fully benchmarked with experiments and analytical formulas and could be implemented in ray-tracing packages as McStas and Vitess. Within this context two main projects are presently in progress:

- A neutron version of the XOP code for x-rays. The result is the NOP (Neutron OPtics) code, already presented in a preliminary version at the SCANS meetings and to be released in April 2003. This code allows computing ab-initio the neutron reflectivity of crystals (perfect, mosaic and curved), mirrors and multilayers (non-graded and supermirrors). These codes are embedded in a friendly visual user-interface and run on multiple platforms. The analytical output might be used as an input of McStas modules.

- Two Monte Carlo (MOSAIC, BENT) codes for fully describing mosaic, curved and gradient crystals. These code have been developed during L. Alianelli's Ph. D. thesis [1], heve been fully benchmarked against theory and experiments and has already been used for the design of neutron instruments. It is being included into the McStas ray-tracing package for use as monochromator/analyser module.

2) Scientific programme

In the frame of the FP6, we intend to continue the existing activity and collaboration among the INFM, the ESRF and the ILL. In this context we will maintain and further develop the NOP, MOSAIC and BENT codes. We will also improve the interfaces of these codes with existing ray-tracing packages (McStas and Vitess) especially in activities relating to file format, routine interfaces and perhaps user interfaces.

This work includes efforts in speeding up the existing Monte Carlo based programs by implementing more efficient calculating algorithms and also by introducing some approximations in the conceptual models.

Moreover, in the frame of the FP6 program we intend to carry out an activity aimed at producing new routines, to be inserted in the existing packages, and devoted to a better physical description of optical elements. In particular we will concentrate on a better description of background. Some problems pointed out in [1] should be investigated, from the point of view of theory and also of the algorithms.

In the case of crystals (monochromators and analysers) the subjects to be improved include:

- Thermal Diffuse Scattering. An accurate description of the TDS is necessary for the simulation of the reflectivity increase of cooled monochromators. It has been pointed out that the usual models do not agree with the experimental results, for the case of mosaic copper (see [1]). Hence, it is necessary to well understand the effect of the temperature on the reflectivity of the principal reflection. Moreover, the implementation of formulas describing the TDS background (which has been studied by other authors, see [2]) will be investigated.

- Parasitic Bragg reflections. When aligning a crystal monochromator it is essential to look for the maximum of reflectivity of the principal reflection by minimising the excitation of multiple simultaneous reflections (the so-called "Unweganregung" reflections). These reflections can be fully calculated and included in the simulations. Their effect is to dramatically increase the effective attenuation coefficient of the incident and diffracted beams in the crystal. This has a strong influence on the integrated reflectivity of mosaic and bent crystals. These spurious reflections are also a source of background.

- Inclusion of magnetic scattering to simulate polarizing crystals.

- Determination of time broadening induced by multiple diffraction events in mosaic crystals (this is of particular importance in the case of pulsed neutron sources).

Regarding multilayer and mirror simulations we propose:

- To develop methods for describing the effect of scattering by rough interfaces.

- Global optimization methods for fitting the supermirror reflectivity and for optimising their design. We plan to adapt methods and codes developed in the x-ray domain.

- To extend the above programs to the case of magnetic multilayers.

Simulation of polarising components:

- Computation of magnetic fields in 3D regions with either analytical approximation or more accurate models (involving slower calculations).

- Starting from the magnetic fields, describe neutron instrument components as flippers, coils, 3He cells.

- Solve the Bloch equations for neutron propagation through the simulated magnetic device (see [3]).

3) Personnel

The staff people involved in this project are:

- Ken Andersen, Head of Neutron Optics Laboratory at ILL

- Emmanuel Fahri, Scientist at the Scientific Software of ILL

- Roberto Felici, Head of the Operational Group in Grenoble of INFM

- Mark Johnson, Head of the Scientific Software of ILL

- Manuel Sanchez del Rio, Deputy Head of Scientific Software of ESRF

Requested funds:

- We ask for a four year scientist position for Lucia Alianelli, currently SCANS Post-Doc (~ 40 kEuros/year).

- A three years Doctorate fellowship (to be recruited, 20 kEuros/year).

- Computers (10 kEuros).

- Travel (30 kEuros)

References

[1] L. Alianelli "Characterisation and modelling of imperfect crystals for thermal neutron diffraction", PhD thesis, Université Joseph Fourier, Grenoble, France, 2002.

[2] C.J. Carlile, M.A. Adams, P.S.N. Krishna, M. Prager, K. Shibata, P. Westerhuijs,

"Less background, better contrast by cooling analyser crystals'', Nucl. Instr. and Meth. A338, 78-82, 1994.

[3] P.A. Seeger and L.L. Daemen, "Numerical solution of Bloch's equation for neutron spin precession'', Nucl. Instr. and Meth. A457, 338-346, 2001.