# The NMI3 JRA on Software: Monte Carlo simulations

Project proposal

Monte Carlo ray-tracing simulation of has proven to be a very reliable way of describing the performance of neutron instruments. In the latest year, almost any new instrument has been thoroughly optimized in software before the actual construction, thus minimizing the risk of expensive reconstruction upon design failures. This has been the case, e.g., for the instruments at FRM-2 and SNS, and it will be the case for future projects like ISIS 2nd target station and ESS.

Monte Carlo ray-tracing simulations have, however, a much larger potential. By simulating a complete experiment, including scattering from a realistic model sample, and data analysis, one may obtain at real “neutron flight-simulator”. With this in hand, much is to be gained before, during, and after a typical neutron experiment:

- The users may gain experience in the use and capabilities of the instrument (perhaps even before applying for beamtime).

- The users and the instrument responsible may investigate the optimal instrument settings for the instrument before a scheduled experiment.

- The instrument responsible is able to perform experimental diagnosis and possible error corrections “on-the-fly”.

- Non-idealities like asymmetric resolution functions, self absorption, small misalignments, etc. may be incorporated by the users in the data analysis.

This JRA proposes to do exactly this: To support the existing Monte Carlo capabilities and turn them into strong and useful tools for all instrument responsibles – and eventually for the every-day neutron user.

In the neutron community today, five different general-purpose ray-tracing packages exist. Two of these (NISP and IDEAS) are American, while three are based in Europe:

- McStas (Risø, ILL)

- RESTRAX (NPI)

- VITESS (HMI)

In this JRA we propose to continue the development of all three packages. The rationale for this is reliability. The latest four years has shown (through the FP5 SCANS network) that comparison between packages is extremely efficient in locating calculation errors, and convergence of the results assure the users of the reliability of the otherwise opague calculations. Another reason for supporting three packages is that the packages are seen to benefit from ideas and tools developed by other packages – and that friendly competition for users has inspired much development towards support and user-friendliness. In economic terms, the development of each package is relatively inexpensive, compared to the cost of a major design failure on, say, an ISIS instrument. (Responsibles: Risø, HMI, NPI)

A code-intercomparison project between all five packages, simulating the same instrument (the Brookhaven triple-axis spectrometer  H8) has recently been finished (P.A.Seeger et al, Neutron News 2002). In the JRA we propose to perform intercomparisons for most types of instruments, and also to compare with the performance of real instruments. The ILL triple-axis spectrometer IN12 and the inverse-geometry TOF spectrometer IRIS at ISIS have been identified as the first targets of comparison. Also the planned (early 2004) PSI instruments MARS and TNT may be candidates. (Responsibles: ILL, ISIS, and PSI)

During the time span of FP6, a large number of TOF instruments are about to be designed (ISIS and ESS). Much effort will be spent on ray-tracing simulations in connection with the design phase, and it is important that the packages are ready for this demand. For optimal use of resources, it is essential to have one or two central contact points for TOF simulations – “simulation help desks” - which also develop and test new simulation modules for TOF. The help desks communicate with both users and package developers to assure that necessary system development is carried out – preferably before the instrument designers need the additional functionality. (Responsibles: ISIS and HMI)

The crucial part in realizing the virtual experiment vision is the existence of realistic sample components. In all packages rather simple samples exist (incoherent scatterer, powder, diffracting crystal), while more realistic, complex samples are still on the whish list. Recently, the concept of virtual experiments was demonstrated by interfacing the Risø instrument control program TASCOM to a McStas simulation for use in data analysis (H. Edwards et al, Appl. Phys. A, 2002). However, much work is still ahead before virtual experiments may become an everyday tool for all neutron instruments. This requires both a development of the simulation packages, a suite of scattering sample components, and modifications to existing instrument control programs. This is by far the most ambitious, but also potentially most rewarding, task in this proposal. (Responsibles: all)

A complementary route to optimization of instrument beam time is intelligent instrument control, where a software algorithm is controlling the data taking of the experiment, deciding, e.g., how much time is needed per measurement, and how many measurements are needed for a complete data series. The feasibility of this concept is realized by the ADAA program on the SLAD diffractometer at UU. We propose a further development of this concept to investigate its usefulness in other types of experiments. The first candidates will be single crystal diffraction and quasi-elastic scattering. A stong aid in the further development of ADAA is the existence of virtual experiments, which will be used for testing the ADAA algorithms on known scattering cross sections. (Responsible: UU)

Of course, the basis for the work is that the packages are supported and that they are continuously developed towards ease of use. One step in this direction is to incorporate the common NeXus data format into all packages in order to ease comparison between codes and with experiments. Further, simulation output may in this way easier interface to existing data analysis tools. (Responsible: PSI)